SFC40A Laser Target Designator & Rangefinder

Technical Specifications

Sheng Fei Guang Su Technology (Chengdu) Co., Ltd.​​

June 2024

# Scope and Application

## Scope

This specification defines the functional requirements, composition, technical parameters, and environmental conditions for the SFC40A Laser Target Designator & Rangefinder.

## Application

This document applies to the ordering, manufacturing, acceptance testing, and delivery of the SFC40A Laser Target Designator & Rangefinder, serving as the contractual basis for procurement.

# Referenced Standards

GJB 150A-2009        Environmental Test Methods for Military Equipments

GJB 151B-2013        Electromagnetic Emission and Susceptibility Requirements for Military Equipmens and Subsystem

GJB 368B-2009        General Requirements for Maintainability of Military Equipment

# Functional and Performance Specifications

## Functional Capabilities

- Laser ranging；

- External synchronized coded Irradiation and internal coded Irradiation；

- Built-in self-diagnostics (power-on self-test, startup self-test, periodic self-test) and status monitoring；

- Power supply protection.

## Performance Parameters

- Laser Type: Non-temperature-controlled semiconductor pumped solid-state laser

- Wavelength: 1.064μm±0.003 μm

- Energy: ≥40 mJ

- Pulse Width: 15±5 ns

- Startup Time	: ≤1 min

- Beam Divergence: ≤0.4 mrad

- Single-Pulse Energy Stability: ≤8% fluctuation (after 2s warm-up)

- Ranging Accuracy: ±5 m

- Ranging Frequency: 5 Hz

- Irradiation Base Frequency: 20 Hz

- Built-in Encoding: 8 preset fixed-frequency codes

- Encoding Precision: ≤±1μs

- Ranging Distance (Ground-to-Ground, Large Targets): 300 m~25,000 m (Visibility ≥23 km, medium atmospheric turbulence, 6 km altitude, target reflectivity 0.2)

- Continuous Irradiation: ≥60 s per cycle, ≤60 s interval between cycles. Conduct continuous irradiation for two cycles.After 2 cycles, subsequent intervals: 30 min

- Continuous Ranging: ≥1 min (5 Hz)

- Parallelism between the laser optical axis and the installation reference plane:≤ 0.6 mrad (laser beam axis/installation base misalignment)

# Interfaces

## 4.1 Electrical Interfaces

- Data Bus: 1-channel RS422 ,communication rate 115,200 bps

- Power Supply: 28 VDC (Operating range: 22–32 V)；

- External Sync Signal: RS422 differential；

- Power Consumption: ≤80 W (peak).

The electrical interface definitions of the product are detailed in Appendix A.

## 4.2 Mechanical Interfaces

- Dimensions: ≤103 mm × 89.5 mm × 50.5 mm (3D model in Appendix B)；

- Weight: ≤535g.

# Quality Requirements

## 5.1 Reliability

Mean Time Between Failures (MTBF): 1,800 hours.

## 5.2 Maintainability

Full-unit replacement within ≤30 minutes.

## 5.3 Testability

Built-in BIT (Built-In Test) with real-time fault detection and status reporting.

## 5.4 Supportability

a) Maintenance tools list, user manual, control software, and cables；

b) Control software, and cables.

## 5.5 Safety Compliance

Designed per GJB 900, GJB/Z99, and GJB/Z102.

## 5.6 Environmental Resilience

- Operating Temperature: -40°C to +60°C；

- Storage Temperature: -55°C to +70°C；

- Max Altitude: 4,500 m；

- Complies with GJB 150x.A-2009 for military environmental testing.

Appendix A

| Connector model (system)：J30JZLN15ZKWA000 |  |  |  |
| --- | --- | --- | --- |
| External connector model：J30JZ/XN15TJCAL01(L=300) |  |  |  |
| PIN | Definition | Remarks |  |
| 1 | 24V+(DC22V~DC32V) | Positive 24V power supply |  |
| 2 | 24V+(DC22V~DC32V) |  |  |
| 3 | 24V+(DC22V~DC32V) |  |  |
| 4 | 24V+(DC22V~DC32V) |  |  |
| 5 | 24VGND | 24V power ground |  |
| 6 | 24VGND |  |  |
| 7 | 24VGND |  |  |
| 8 | 24VGND |  |  |
| 9 | RS422 Data Reception+ | Laser Receiver for RS422 |  |
| 10 | RS422 Data Reception- |  |  |
| 11 | RS422 Data Transmission- | Laser Transmitter for RS422 |  |
| 12 | RS422Data Transmission+ |  |  |
| 13 |  |  |  |
| 14 | External Time Signal+ | RS422+ |  |
| 15 | External Time Signal- | RS422- |  |

Appendix B